Document Authorization:

|  | Name | Date | Signature |
| --- | --- | --- | --- |
| Owner | Marites Erfe | 30Apr2019 | See Signature on File |
| Operation Management | Xibo Li | 01May2019 | See Signature on File |
| Operation Management | Pierre Barratt | 01May2019 | See Signature on File |
| Quality Assurance | Josephine Gammad | 01May2019 | See Signature on File |

Changes from previous version:

| Section | Summary of Changes | Change Control Number |
| --- | --- | --- |
| All, 3 | Updated document number to FMP numbering system. Added Phase II form. | DCR19014 |
| 5 | Added back Production Management and Production Staff in the responsibilities section. Quality Assurance – change support to approve. |  |
| 6 | Added Phase II and Phase III in the definition section. |  |
| 7.3 | Added Phase II instructions. |  |
| 7.4 | Added Phase III instructions |  |

- PURPOSE

This procedure describes a procedure for the investigation of out-of-specification (OOS), out-of-expectation (OOE) and out-of-trend (OOT) results that are obtained during analysis.  OOS, OOE and OOT results are referred to collectively as OOX results in the rest of this document.

- SCOPE

This procedure applies to all GMP test results generated in LGC’s Analytical Testing Laboratory in Petaluma, CA, including but not limited to;

- Release tests for bulk and finished products, active pharmaceutical ingredients and intermediates

- Incoming goods testing for raw materials, packaging materials

- In-process controls (IPC) forming the basis for a quality evaluation

- Analytical tests connected to manufacturing process validation

- Stability tests for bulk and finished products and active pharmaceutical ingredients.

Out of Scope:

This procedure does NOT apply to results generated from the following activities:

- In-process testing not used directly as basis to evaluate quality

- Method development

- Method Transfer and method validation

- Early formulation or process evaluation

- Cleaning verification

- Early candidate support

- Active Pharmaceutical Ingredients where information is being gathered upon which specifications or targets are being based.

- Processed Product moved in-house.

- INTERNAL REFERENCES

| Document ID | Title |
| --- | --- |
| POL-208849 | Quality Policy |
| SOP-208827 | Quality Assurance Audit Program |
| SOP-208833 | Risk Management |
| SOP-208890 | Quality Event Investigation |
| FM-208963 | Quality Event Investigation Form |
| FM-208965 | Preliminary Laboratory Investigation Report |
| FM-208966 | Phase I Laboratory Investigation Report |
| FM-208967 | Phase II Laboratory Investigation Report |

- EXTERNAL REFERENCES

| Document ID | Title |
| --- | --- |
| ICH Q7 (API) | Good Manufacturing Practice Guidance for Active Pharmaceutical Ingredients (API) |
| ICH Q9 | Quality Risk Management |
| ICH Q10 | Pharmaceutical Quality System |
| ISO 9001:2015 | Quality Management System Requirements |
| FDA Guidance for Industry October 2006 | Investigating Out-of-Specification (OOS) Test Results for Pharmaceutical Production |

- RESPONSIBILITIES

| Job Function and/or Department | Responsibility |
| --- | --- |
| Business Development Representative and/or Project Manager | Notifies customers of any identified OOX and the progress of the investigation. |
| Laboratory Management | It is the responsibility of Laboratory Management to initiate and notify QA of the Preliminary Laboratory Investigation (PLI) within one business day after discovery. It is the responsibility of Laboratory Management to facilitate and assure that the Laboratory investigation (Phase I) is carried out within 10 business days once PLI is initiated. It is the responsibility of Laboratory Management to notify the QA and the sponsor with one business day (or a time as defined in the TQA) and initiate a quality event investigation within one business day of completion of the Phase I investigation if no clear laboratory error has been identified. It is the responsibility of Laboratory Management to facilitate and assure that the quality event investigation FM-208963 is carried out within 30 business days from when quality event investigation is initiated. |
| Production Management | It is the responsibility of Production Management to initiate a Phase II Investigation within 24 hours if no clear laboratory error has been identified during Phase I investigation. It is the responsibility of Production Management to facilitate and assure that the Manufacturing investigation is carried out within 30 business days from when OOX was identified. It is the responsibility of production management to notify the laboratory management, QA, the BD representative and/or customer within 24 hours of the conclusion of completion of the phase ii investigation. |
| Laboratory Staff | It is the responsibility of laboratory staff to ensure that OOX are reported to management and are logged in the quality events system as soon as possible, but no more than one business day after discovery. It is the responsibility of laboratory staff to complete the preliminary laboratory investigation within one business day of discovery. It is the responsibility of Laboratory Staff to initiate and complete the Phase I Laboratory investigation within 10 business days once OOX is identified. It is the responsibility of the Laboratory Staff to complete the quality event investigation FM-208963 within 30 business days |
| Production Staff | It is the responsibility of production staff to facilitate the completion of the phase ii manufacturing investigation within 30 business days once OOX is identified. |
| Quality Assurance | It is the responsibility of QA to evaluate the OOX result and assess severity (critical, major, and minor) and impact of the OOX on the quality, validation and regulatory status and assign investigation to Laboratory Management if no clear laboratory error has been identified. It is the responsibility of QA to approve Phase I and quality event investigation. It is the responsibility of QA to monitor OOX and evaluate the preventative and corrective actions. Additionally, OOX are reviewed as part of the Quality System review as per SOP-208827. Customer agreements must be reviewed by QA to ensure that their requirements are met on reporting of the OOX. Quarterly Quality System review of all OOX investigations as per SOP-208827. |

- DEFINITION

| Term | Definition |
| --- | --- |
| Out of Specification (OOS) | A result that falls outside of approved product specifications. |
| Out of Expectation Result (OOE) | A result that is within specification but shows poor agreement, deviates from the theoretical and/or does not conform to trends observed in previously generated data. |
| Out of Trend Result (OOT) | All reportable results that are within specification but differ from the data primarily expected in the stability analysis. |
| OOX result (OOX) | Synonym for OOS, OOT and/or OOE results. |
| Laboratory Investigation Report (LIR): | Documentation of the investigation to identify the validity and cause of a Quality System Review or OOS laboratory result. |
| Attributable / assignable Cause | A laboratory error with a clear and scientific reason for the result. |
| Un-attributable Cause | A laboratory error with no clear scientific reason for the result obtained. |
| Preliminary Laboratory Investigation (PLI) | Determines whether there has been a clear obvious error due to external circumstances that will negate the requirement of a Phase I investigation. For e.g., power outage, calculation error, incomplete transfer of sample, incorrect instrument parameters. |
| Phase I Investigation | Conducted by the analyst and supervisor when the PLI does not determine a clear obvious error. The Phase I investigation is performed to determine whether there is a clear and obvious attributable cause. Investigation should be restricted to data / equipment / analysis review only – No re-measurement should be performed. Only if an attributable cause is identified, the sample analysis may be repeated, and the original result can be discarded. Examples can include correct test methods followed e.g., version number, correct sample test, possibility of sample contamination, equipment within calibration, standard / controls performed as expected etc. |
| Phase II Investigation | It is carried out when an assignable laboratory error has not been identified from the Phase I Investigation. The Phase II investigation is to be driven by written and approved instructions against hypothesis. It may also include the initial manufacturing investigation to determine whether there was a possible manufacturing root cause. Additional testing using a predefined plan may include retests performed by a second analyst with equivalent experience in the method. |
| Out of Specification (OOS) | A result that falls outside of approved product specifications. |
| Phase III Investigation | Is performed to conclude all the investigations (including manufacturing) and results that have been generated. The batch quality must be determined and a decision made upon the disposition. A review of impact on all other batches, stability studies, validated processes and test methods should be documented. As part of the Phase III investigation, a monthly review of all OOS/Quality System Review investigations will take place by the Quality Manager. This will be used to identify any trends and determine the effectiveness of any CAPA. |
| Re-Test | Performing the test over again using material from the original sample, if it has not been compromised and is still available. |
| Re-Sample | A new sample from the original container where possible, required in the event of insufficient material remaining from original sample composite or proven issue with original sample integrity. |
| Most Probable Cause | Scientifically justified determination that the result appears to be laboratory error. |
| Hypothesis / Investigative Testing | Testing performed to help confirm or discount a possible root cause. |

- PROCEDURE

  - Preliminary Laboratory Investigation (PLI)

    - A preliminary laboratory investigation (PLI) is conducted by the laboratory staff to determine whether there has been a clear obvious error due to external circumstances for e.g., power failure, sample spillage, incomplete transfer of sample, incorrect instrument parameters, clear calculation error etc.  The PLI is recorded using Preliminary Laboratory Investigation Report (FM-208965).

    - A Quality Event should be initiated within one business day and the completed PLI Report should be attached.  Refer to the quality event number in the Laboratory Notebook and/or other relevant raw data.

    - Upon discovery of an OOX results, all samples, subsamples and dilutions of samples associated with the OOX analysis should immediately be placed in a suitable storage container and labelled with reference to Quality Event number.

    - If a clear, obvious error is identified this will negate the requirement of a Phase I investigation.  This will be clearly documented in the Laboratory Notebook and/or other relevant raw data.  The results are to be invalidated with reference to the Quality Event number and the analysis repeated.  Clear obvious calculation errors need to be reviewed, signed and dated, by both laboratory staff and Laboratory Management.

    - If no clear, obvious error is identified during the PLI, a Phase I investigation is conducted by laboratory staff and Laboratory Management and recorded using Phase I Laboratory Investigation Report 
(FM-208966).

    - Laboratory Management will notify QA and the sponsor within one business day (or a time as defined in the TQA) of completion of the PLI if no clear laboratory error has been identified.

    - The PLI Report must be reviewed and approved by the Laboratory Manager.

  - Phase Investigation

    - The Phase I investigation is conducted by the analyst and the Laboratory Management and includes assessment of data, sample, method procedure, method performance, training reagents and equipment used.  The Phase I investigation is recorded using Phase I Laboratory Investigation Report (FM-208966).

    - An immediate assessment could include visual examination of the actual solutions, test units, and glassware used in the original measurements and preparations.

    - The Laboratory Management will discuss the test method with the analyst to confirm analyst knowledge of and performance of the correct procedure.

    - Raw data obtained during the analysis, including chromatograms, spectra, audit trails will be assessed and calculations used verified, performance of the instruments used assessed.  Preparations of reference standards, solvents, reagents, and other solutions were used will be checked and confirmed that they met quality control specifications.  Assess the performance of the test method to ensure that it is performing according to the standard expected based on method validation data and historical data.  This investigation will be clearly documented in the Phase I investigation report (FM-208966). The Phase I investigation report is signed by the laboratory staff and approved by Laboratory Management and QA.

    - When a laboratory error can be identified and clear evidence of the error exists, the laboratory testing results should be invalidated and will be clearly documented in laboratory notebook and/or other relevant raw data with reference to the Quality Event number and the analysis repeated.  The analysis should be repeated on the sample or a new sample (if the investigation shows there to be an error with the original sample) and original result invalidated.

    - When a laboratory error cannot be easily identified but on completion of the investigation, a most probable cause has been identified, hypothesis testing can occur with an approved pre-defined test plan to confirm this most probable cause.  Description of the testing should be written in a Phase I hypothesis testing plan (FM-208966) and then approved by Laboratory Management and QA prior to initiating investigational testing.  This hypothesis testing may continue from the re-measurement of the original preparations. Investigational testing may not be used to replace an original suspect analytical result.  It may only be used to confirm or discount a probable cause. It should be suitably designed to allow investigation per one possible variable.  The hypothesis plan should contain as a minimum:

- a.    The hypothesis of the probable cause being investigated

- Samples to be tested

- Details or reference to the testing method

- Evaluation of the test results and/or decision tree

- When the hypothesis testing confirms a laboratory error, the laboratory testing results should be invalidated, and the analysis should be repeated on the sample or a new sample (if the investigation shows there to be an error with the original sample) and original result invalidated.

    - When evidence of laboratory error remains unclear, a full-scale investigation should be conducted by the production staff and management to determine what caused the unexpected results using form FM-208963, Quality Event Investigation Form.

    - QA must be informed of the outcome of the Phase I investigation.  The original results and raw data must be retained with reference to the Quality Event number.

    - Corrective/preventative actions should be identified to ensure that the same errors are not repeated in the future.  These actions are to be recorded within the related Quality Event with timelines included as to completion.

  - Phase II Investigation

- When the Phase I Investigation does not determine that laboratory error caused the OOS result and testing results appear to be accurate, a full-scale OOS investigation will be conducted by production staff, management, and QA.  The Phase II investigation consist of a review of production and sampling procedures, production process review and/ can include additional laboratory work being conducted according to a written and approved testing instruction / hypothesis, developed to identify the root cause of the OOS.

    - The Phase II investigation is conducted by the production staff and Management and includes review of the manufacturing process, sampling process, equipment, reagents used and documentation review.  The Phase II investigation is recorded using Phase II Full-Scale OOS Investigation Report.  The Production Management will discuss the manufacturing process with production staff to confirm production staff knowledge of and performance of the manufacturing process

    - When a sampling error can be identified and clear evidence of the error exists, this will be documented on the Phase II Investigation report and approved by Production management and QA.  A new analysis must be conducted using a new sample according and analysed according to the test specification.  Prior approval by the releasing QA function.

    - When an error can be identified as part of the batch evaluation and clear evidence of the error exists, the initial OOX result is regarded as confirmed.  This indicates that the batch does not meet the establish standards, expectations and/or specifications.

    - If no root cause for the initial OOX result is found during phase II, the OOX investigation must be extended to include the API manufacturing process.

      - If no attributable cause that could explain the results can be identified during the manufacturing investigation or the initial laboratory hypothesis investigation, re-testing may be considered.  Re-testing must be performed as per documented and approved testing plan using the original sample, maybe a second aliquot.  If it is not possible to use the original sample the resampling process (see section 7.3.4.1) must defined and approved by QA.  The number of retests must be statistically valid, based on either method validation or % RSD:  suggestions: 5, 7, or 9.  In addition, to be carried out by different analyst where possible, with similar or more experience in the method / instrument as the original analyst. The re-testing plan should detail:

  - Samples to be tested

  - Details or reference to the testing method

  - Evaluation of the test results and/or decision tree

  - Results generated during the investigation process must not be used as the reported results

      - Re-Sampling (collecting a new sample from the batch)

- Only, when necessary, if insufficient quantity of original sample remains, original sample was not truly representative compromised or invalid.

- To be performed after discussion with Quality, using same process as original sample (unless process at fault)

- Sound scientific judgement to be used.

- Sampling method to be fully documented.

      - If hypothesis testing identifies an attributable cause, then the sample can be retested and the original result invalidated.

      - All results are reported to the customer along with the completed investigation report.  Statistical outlier analysis may be included in the laboratory investigation report, however, may not be justification for rejection of data for previously validated chemical tests demonstrating small variance.

    - Corrective/preventative actions should be identified to ensure that the same errors are not repeated in the future.

    - All the information obtained from the investigation will be attached to the investigation for review by QA.  This is to ensure that correct root cause and CAPAs, if any required, have been properly assigned and implemented. This will include an assessment as to the wider impact of the OOS.

  - Phase III Investigation Summary

    - The Phase III investigation summary should conclude all of the investigations (including manufacturing) and results that have been generated. The batch quality must be determined and a decision made upon the disposition. This will be documented in FM-208963, Quality Event Investigation form per SOP-208890, Quality Event Investigation.

    - Should a decision be made to reject the batch it cannot be reversed based on further testing.

    - A review of impact on all other batches, stability studies, validated processes and test methods should be documented.